Islet Quality Assessment

Clark K. Colton

Department of Chemical Engineering, Massachusetts Institute of Technology; Cambridge, MA

Improvement in islet quality assessment requires development of meaningful quantitative assays. In this presentation, the current state of the art and new methods under study are reviewed. One important area is islet enumeration. Studies in our laboratory have shown that nuclei counting, together with microscopy, can provide accurate estimates of the islet content in a preparation. Islet viability is best assessed by assays of mitochondrial function. Prominent in this regard is oxygen consumption rate (OCR), which is a measure of the rate of oxidative phosphorylation. We have made extensive use of the Instech stirred chamber, which provides rapid, accurate results. OCR measurements are predictive of transplantation outcome in immunodeficient diabetic mice when transplanted with rat islets and with high purity (> 90% DTZ) human islet preparations.